Setting up a tftp server.

12/4/2007

Updating the E310/E610 firmware from the CLU or CLI requires setting up a tftp server on the computer that the E310/E610 will be downloading the files from. This document explains how to set up a tftp sever for Windows, SLES10 and Fedora 5.

Note, firmware can be upgraded from any supported browser if  WebPAMPro is used. To connect to WebPAMPro simply input the IP address of the ETrak’s management port into the browser’s URL bar, for example, http://192.168.1.137
Windows

For Windows XP and up, no tftp server is included. There are several free tftp servers that can be used. One is the Solarwinds tftp server which can be downloaded from…
http://www.solarwinds.com/products/freetools/free_tftp_server.aspx
Another free tftp server is TFTPDWIN

http://www.freedownloadscenter.com/Network_and_Internet/FTP_Server_Tools/TFTP_Server_TFTPDWIN_Download.html
Download the firmware files into a directory of your choice then configure the tftp server’s download directory with the location of the firmware image files. If everything is done correctly ptiflash should successfully download and install firmware image files.

Fedora Core
Fedora core does include a tftp server, but it is not furned on by default. To run the tftp server, first configure it. The configuration file can be found at /etc/xinetd.d/tftp

By default the tftp server is disabled.

# File /etc/xinetd.d/tftp

#

service tftp

{

...

...

       server_args             = -s /tftpboot

       disable                 = no

}

Be sure to set disable to no and note the download directory, this is where you need to place your files.
Once the configuration file is set, start the tftp service. You can use the services GUI or

# chkconfig tftp on

SLES 10
With SLES 10 the tftp server is included in the install DVD, but it’s not installed by default. To install it start YAST, enter tftp into the filter window, and hit search. You will get a tftp icon, double clinking on the icon will cause YAST to install the tftp package, which is found on the install media. Note the tftp directory, the default is /tftpboot.
